The  FPGA  Implementation of Viterbi Algorithm in Grand Alliance ATSC Systems.

Assistant Professor ,Dept of ETC, M.I.T.(Aurangabad),Maharashtra.  Mr.D.L.Gadhe.

P.G Student,(Embedded Systems), M.I.T.(Aurangabad),Maharashtra. Ms.S.S. Deshpande.

ABSTRACT: Digital television has replaced analog television system and has evolved as a technique for transmission of high quality video and audio. As ability  to modify picture resolution is limited in analog technique. In ATSC system we can use modulation scheme of increased  level and furtnher advancement or improvement in ATSC system was implemented in 2013 in U.S. Improved version of ATSC is ATSC3.0 system. ATSC 3.0 has an application in television broadcasting as well as mobile communication. Viterbi Algorithm invented by Andrew J. Viterbi  is an heart of  ATSC3.0 system .Viterbi decoders are used in more than one billion cell phones. Convolution coding and Viterbi decoders are most popular choice for error correction and are widely used because of the advantages and ability to work at low S.N.R. In this project we are focusing on channel coding circuit, responsible for addition of redundancy to correct errors. In convolution coding number of output bits obtained is double the input bits if rate is ½. We have observed the synthesis results i.e RTL schematic layout of various modules of Vierbi Decoder on  Xilinx ise 9.1 and also observed the simulation results of the forward error correction circuitary. For Source coding we have used MATLAB and computed DCT and IDCT of the images. Source coding is required compression of images, for transmitting large amount of data with minimum number of bits, and utilization of bandwidth resources efficiently.

INTRODUCTION :In improved ATSC 3.0 system, we are using subcarrier transmission along with 8 VSB modulation that is single carrier multitone modulation scheme.In SCMT modulation we have upsample unit at transmitter and downsample  unit at receiver ,FFT is applied by mapping upsampled charecters in frequency domain and then multiplying it by coefficient. Subcarrier contains the information to be transmitted, subcarrier having frequency value above the carrier frequency is plotted in lower part of the spectrum and subcarrier having frequency below the carrier is plotted in the upper part of spectrum .Along with 8 VSB frames ,SCMT frames are also transmitted ,so that information can be perfectly recovered and multipath interference can be avoided ,single carrier multitone modulation has same principle that of the orthogonal frequency division multiplexing. Subcarriers are orthogonal to each other that is 90 degree phase shift, to avoid the loss of information bits due to multipath interference, for 8 VSB modulation 2 bits are converted into 3 bits data and this three bits of data is mapped to the specific voltage level, which constitute vestigial sideband signal, this signal is multiplied by carrier signal. Information is contained in upper and lower sideband,  these are mirror images of each other, so only upper part of spectrum

Figure1:-8-VSB modulator using weaver method.

called vestige is transmitted along with pilot carrier i.e suppressed  carrier. Single sideband modulation is generated by balanced modulator along with low pass filter. Balanced modulator circuit consist of two FET with similar parameters.FET is having nonlinear charecteristics after some point. Modulating signal and carrier are given as an input to the FET. Output of balanced modulator is a single sideband which is given as an input to the low pass filter. In ATSC3.0 (Advanced Television System Committee with advanced physical layer) symbols generated by quadrature amplitude modulation scheme are upsampled and converted into the charecters .Quadrature amplitude modulation along with 8 –Vestigial sideband modulation, is used in latest version of ATSC3.0. Maximum amplitude levels for Vestigial sideband modulation is 64 whereas Quadrature amplitude modulation is having amplitude levels upto 4096, thus increasing the throughput.

Source coding and Channel Coding:

Source coding techniques are used for compression of video frames and images to transmit video and audio with as few bits as possible. ATSC and ATSC3.0 employees MPEG-2 standards(Moving Pictures Expert Group).For video coding we are employing spatial and temporal redundancies which exist between the frames ,there is small difference between the frames which is predictable. Video screen can be considered as set of scan lines ,each scan lines consist of pixels ,higher the number of pixels representing image higher the resolution.

Each video frame is divided into the 8*8 blocks we can also include the depth parameters i.e insert zeros .DCT(discrete cosine transform) of each 8*8 block is taken proceeded with quantization, bits which survivest the quantization are transmitted in the zigzag format. At the source decoder we can take inverse discrete cosine transform. Complex compression algorithms are included in ATSC3.0 system  that data frames can be represented with minimum bits, along with good quality reception. Differenced image i.e difference between current and previous frame, 16*16 length blocks are considered in MPEG coding, differenced image is called motion vectors, after applying DCT on motion vectors ,they are quantized and scanned in zigzag manner, then run length coding and Huffman coding is performed on zigzag scanned pattern. Both are lossless compression schemes.

advanced forward error correction schemes enable decrease in bit error rate. Reed Solomon and convolution coder or trellis encoder and Viterbi decoder are used for error correction. Reed Solomon codes along with convolution codes gives excellent error correction performance, due to addition of extra redundant bits. Combination of both the codes is mostly used in deep space communication. As constraint length increases bit error rate decreases, forward error correction circuitary i.e convolution encoder and viterbi decoder has better error correction performance at higher constraint length. We can also Viterbi decoder as FSM(finite state machine) for higher constraint length.

Figure2:- Convolution encoder block diagram.

CONVOLUTION ENCODER:

Convolution encoder consist of registers and modulo two adders. Each modulo two adder present in convolution encoder is associated with generator polynomials. Modulo two addition of generator polynomials and input message bits is computed and we have resultant encoded bitstream  having double the length of input message bits. If we are inputting 4 message bits to encoder it will output 8 bit word and so on because we have ½ rate encoder. We also have commutator switch .It  can also be  compared to digital  filter used for signal processing. Generator polynomials related with modulo two adders are created according to the inputs given to the modulo two adders. Encoder containing two D flip flops can be represented by registers i.e we can replace flip flops by registers. Convolution encoder can be represented by trellis diagram, related trellis diagram can be constructed using state transition table. Considering the four state a ,b ,c ,d represented by 00 ,01, 10, 11. When we are giving input 0 to the encoder, state transition is from 00 to 00 ,and when we are giving input 1 to the encoder transition is from 00 to 01.Transition from a  to b is represented by dotted line as input is 1.From reference of transition table we can construct trellis diagram and state diagram .In this manner we can represent convolution encoder by trellis diagram and state diagram.

VITERBI  DECODER:

Viterbi Decoder is used to decode the message sequence from the encoded sequence obtained from convolution encoder series of trellis structure connected with each other followed by tail bits is used to decode the message sequence. Last two stages of trellis structure are forced to be zero as we are also transmitting synchronous bits. So for the convenience we are forcing last two bits to be zero. Received bit stream may contain errors ,but due to interleaving and deinterleaving  burst errors are avoided viterbi decoder has error correction capability of 2 and is dependent on constraint length. Error correction capability is improved at the cost of increase in decoder complexity and increase in storage. Decoder complexity increases with increase in length of received  sequence initially grouping of trellis is done, all the branches corresponding to 00 in the trellis are xored with the received sequence and hamming code related with each branch is computed and similarly considering next group of branches i.e branches corresponding to 01 are considered and hamming  codes corresponding to these branches  are considered and each branch corresponding to 01 is allotted a branch metric value i.e difference between  received sequence and 01 branch group of each stage is taken, each 01 branch is allotted a branch metric value. Similar procedure for other two groups i.e groups corresponding to 10 and 11. Next step is to assign partial path metric values to nodes present in the trellis, each node is assigned a value based on previous branch connected to it ,if single branch is connected to the node then value assigned to node is

Figure3:-Principle of Viterbi Algorithm

equal to that of a branch whereas in next stage of trellis two branches are input to the node, in such situation comparator selects the branch with minimum metric value, that is summed up with the value of previous node present in the row ,selected branch among the two is indicated by an arrow ,node values are computed column wise i.e initially we are computing values of nodes related with first column then second and so on. Selection of branches is done in such a way that minimum partial path metric values are allotted to the nodes. For selection of maximum likelihood path, we are considering the nodes with minimum values along with the branches which are selected and thus indicated by arrows ,maximum likelihood path is the survivor path i.e path with minimum hamming distance.

The selected path is  traced back with time, we are considering the trace back approach to avoid the extra switching activity and reduce the storage space , in this approach we are storing only required path i.e maximum likelihood path, instead of storing every detail related with sequence.

In register exchange method each node in trellis is allotted with register, for storing the computed , plus additional registers for storing survived paths having values near to the maximum likelihood path and final path. Trace back method allows reduction in storage space .At time instant t7 last branch of maximum likelihood path is traced, and according to the state transition of the branch traced back ,we retrieve the source bit associated with that branch. Each branch is associated with source bits 0 or 1 , these source bits related with the branches constituting maximum likelihood path is decoded bit stream, which we have inputted to the convolution encoder .At time instant t6 second last branch is traced back. In this way branches selected are traced till pointer arrives at initial state. This is the conventional method for decoding of convolution codes using Viterbi Algorithm.

VITERBI  ALGORITHM:

Viterbi Algorithm was invented by Italian American electrical engineer Andrew J. Viterbi it is called optimum algorithm, as it perfectly corrects the errors and is suitable for convolution codes having constraint length less than 10.It is divided into three steps.

1)Calculation of branch metrics related with each branch.

2)Comparsion of branch metrics through ACS(add compare select unit) and finding of survivor path from initial state to final state.

3)Tracing back the selected maximum likelihood path and based on transitions included in the maximum likelihood path decoding the sequence to produce decoded bitstream.

Justification of Viterbi Algorithm:

Various decoding techniques were introduced before viterbi decoding. From comparative analysis and application requirements we may conclude that VA is optimal choice for constraint length below 10, with efficient error correction capability .Comparative Analysis:

1)VA is having good synchronization and convienience can also be achieved by introduction of tail bits .Symbol arrival rate is made equal to the symbol decode rate. Even if mismatch all states are initials to 0 and reception of inputs is automatically stopped till synchronization is achieved hence loss of information bits for retrieval of decoded sequence is avoided. There is no requirement for retransmission of bitstream.

Whereas in sequential decoding schemes synchronization is not achieved easily because of its complexity and absence of tail bits if symbol arrival rate does not match symbol decode rate buffer overflows, causing loss of information.

2)Viterbi algorithm is not based on trial and error method. Results can be achieved in one iteration if implemented in efficient manner.

Sequential decoding is trial and error method.

3)Viterbi Algorithm works effectively even in the cases of reduced SNR, due to inclusion of interleaving and deinterleaving scheme .

In sequential method due to reduced SNR number of poor hypothesis increases and error rate also increases.It is suitable for higher values of signal to noise ratios, thus reducing the error correction capability.

4)In VA by considering only maximum likelihood decoding and including decision unit number of iterations are reduced to 1 .

In Sequential method number of iterations are increased if value of sequential path is greater than prescribed threshold value.

5)VA requires less amount of storage as we are avoiding comparison of received sequence and previous sequence.

In sequential method comparison of previously selected branch is done with next set of received  symbol to get the decoded sequence.

6)It can be implemented in parallel to increase the through put rate by increasing the number of  ACS (add compare select)units in the device .Soft decision mode can be easily included and hence bit error rate is reduced to the great extent.

In sequential algorithm soft decision decoding cannot be used as complexity it increased to higher extent already sequential methods are complex and soft decision will increase it further. Traceback length and storage required for sequential  decoders is increased.

ARCHITECTURE  OF VITERBI DECODER:

Viterbi decoder consist of four major parts BMU(branch metric unit),PMUS(path metric update sequence),ACS(add compare select),ISOD(information sequence output decision).

Branch metric unit:

This unit computes hamming distance in case of hard decoding and Euclidean distance in case of soft decoding.  It uses the received sequence and encoded sequence related with trellis stages to compute hamming code. It performs differance  and addition operations and computes branch metric and partial path metric values corresponding to each branch in the trellis stages.

Path metric update sequence:

Output of branch metric unit ,i.e branch metrics computed are given as an input to the path metric unit ,it adds the current branch metric value to the previous branch metric value and computes partial path

Figure4:-Block diagram of Viterbi decoder.

metric value. The branch values of corresponding branches must be connected with each other .In this manner it computes partial path metrics from initial stage to some intermediate stage.These partial path metric values are given as an input to add compare select unit.

Add compare select unit:

It consist of comparator unit .Comparator unit selects compares the path metric value and selects the one with lesser magnitude. Path having path metric value lesser is almost equal to the received sequence. All the paths including competing paths along with survivor path correspond to a particular encoded sequence .Encoded sequence which matches the received sequence is selected, survivor path i.e the selected path is having minimum hamming distance value .All the branches in the survivor path are connected.

Input sequence output decisions:

This is the last unit of Viterbi decoder ,it outputs the decoded sequence on basis of selection of maximum likelihood path, in trace back approach at each time instant a particular branch is traced back thus giving the state transition associated with  particular branch which is traced back at particular time instant say t7. Each state transition has corresponding source bits attached to it. These source bits can be either 0 or 1, source bits connected with maximum likelihood path and outputted as the decoded bitstream. In conventional Viterbi decoder we are using shift registers in which data bits are shifted from left to right ,in LIFO pattern last in first out method. In this way we get serial input message bitstream at the output of Viterbi decoder. We can compute the bit error rate by inputting Viterbi decoder output to the convolution encoder and encoded output bitstream is compared with received bit stream containing few erroneous bits. We can compute hamming distance between encoded bitstream (original) and received bitstream ,number of ones in hamming code will indicate position of erroneous bits, and also give us the bit error rate. Lesser number of ones indicate less number of errors, whereas more number of ones indicate increased error rate. Bit error rate is reduced to great extent if constraint length is increased and also there is 3 db reduction BER due to inclusion of soft decision Viterbi decoder but with increased complexity and storage space.

PARALLEL PIPELINED VITERBI DECODER:

Modern communication system requires devices providing high speed, high throughput and low power consumption .Power consumption and speed depends on ACS (add compare select units) and trace back units .In pipelining scheme instructions are executed simultaneously ,larger number of computations are performed in single clock cycle. This computation is similar pipelining applied in DSP processor. In single clock one instruction is fetched another is decoded and third is executed .In parallel Viterbi decoder there is overlapping of input bitstream ,when one input bitstream is compared and in second clock cycle one of them is selected and in same clock cycle other bitstream is given as an input to the same comparator unit .In this manner

Figure5:-Selective update registers arranged in parallel Viterbi decoder.

Figure6:-Deep pipelined execution of  add compare select unit in Viterbi decoder.

Figure7:-Deep pipelined execution of the trace back unit.

throughput can be increased in parallel Viterbi decoder .Clock  frequency is directly proportional to the length of maximum likelihood path, greater the path length greater the clock frequency required for Viterbi decoder .High clock frequency leads to increase in power consumption, to reduce the power consumption in Viterbi Decoder ,maximum likelihood path length is reduced.

Reduction in survivor path length leads to reduced clock frequency and hence power consumption. Routing and connecting wires required for connecting registers are reduced in parallel Viterbi decoder as contents of registers cannot be changed and shifted in parallel Viterbi decoder. In conventional Viterbi  decoder shift update scheme is used and data is shifted starting from LSB(least significant bit) and registers are arranged horizontally. Whereas in parallel Viterbi decoder selective update scheme for registers, in selective update registers are arranged vertically, vertically arranged registers are equal to the number of state columns  .Each stage has 8-bit registers, after completing  survivor path calculation clock of registers is enabled and corrosponding maximum likelihood path is stored in registers along with source bits .Here we are introducing clock only when blocks used in Viterbi decoder circuit are in active mode(performing computation) when blocks are in passive mode(not working) clock is disabled .Clock gating scheme in parallel Viterbi decoder has reduced power consumption to the great extent as clock input is not given continuously .As register contents in selective update cannot be changed and presence of D-flip flop instead of register has reduced the switching activity and due to reduced switching between registers ,power consumption is again reduced. Hence we have efficient Viterbi decoder with increased throughput and reduced power consumption using FPGA, suitable for modern communication system.

SIMULATION  RESULTS :

In parallel Viterbi decoder we are reducing the number of states and thus power dissipation. Viterbi decoder is separated into several small clusters and each cluster is having same number of states. We observed rtl schematic and technology schematic using Xilinx ise 9.1 .If we implement via compare select add then number of comparators are increased ,and hence decoding speed. We observed the simulation results on Xilinx ise simulator and also the hamming distance between received code word and encoded sequence. Length of the traceback is also reduced hence pointer to store the traceback  in memory location .

Figure:-RTL schematic of the Parallel Viterbi Decoder.

Figure:- Technology schematic parallel VD.

Figure:-Package pin layout top view of the FPGA device xc3s200.

Figure:-UCF file with layout and locations of LED assigned to the decoded sequence.

Figure:-Received sequence assigned to the the pins connected with switches.

Figure:-Timing summary of parallel Viterbi Decoder with three stages.(per cluster)

Figure:-Power summary in mw of  parallel Viterbi Decoder .

Figure:- Simulation results showing comparison between path metric values of  two survivor paths.

Figure:- Simulation results displaying hamming distance computed by branch metric unit.

8VSB MODULATION RESULTS:

Figure:-Vestigial sideband signal.

Figure:-Simulation results of maximum    likelihood path selection through path metric.

Figure:-Modulating signal corresponding    to  Vestigial signal.

Figure:-Set of Carrier signals for   transmitting information content in modulating signals.

Figure:-VSB Modulated signals with upper and lower sideband with orthogonal carriers.

Figure:-Resultant vestigial single side

band signal obtained after cancellation

of two out of phase carriers.

IMAGE COMPRESSION RESULTS:

Information is compressed before further

Processing it to represent in as few bits

as possible, present and future picture fr

ames are difference and motion vectors

are obtained.

Figure:-Discrete cosine transform of

Camera man image.

Figure:-Individual and improved DCT of

Image appended with zeros and segmented

Into 8 *8 blocks.

Figure:-Improved IDCT of segmented block

8*8 computed at MPEG source decoder.